Version 2026-04-01 11:11, Eric Pohl

## Instruments

All instruments commonly used for hydrological measurements by Uni Fribourg in Central Asia used are explained in the following; a summary of these instruments is provided in Table 1. While a specific instrument might use local time zones, it is good practice to save all data, always in UTC+0 time zone. This makes using data also from other places much easier.

### Water Level

### Two types of instruments are used by Uni Fribourg in Central Asia: 1) Onset HOBO U20 and U20L 0-4 m Water Level Loggers, and 2) van Essen Diver DI801 0-10m Data Loggers. Due to various problems and very expensive repair costs (instruments need to be shipped to the USA), Uni Fribourg is not renewing the HOBO loggers but is instead transitioning to van Essen loggers. Both instruments are operated with a 1 h temporal measurement interval starting at the full hour in units of kPa (HOBO) or cmH2O (van Essen). This allows for a good capture of dirunal variability. Their accuracy (resolution) is 0.3 (0.14) cmH2O (HOBO) and 0.5 (0.2) cmH2O (van Essen). The sensor pairs (water and atmosphere) are started always on a same computer to ensure synchronous data records. Time zones are recorded in the stored data files.

### Fluorometer

An Albillia GGUN-FL30 fluorometer (FL30) is used at all sites to obtain single point in time (snapshots) estimates of discharge using Sodium-Fluorescein as fluorescent dye tracer. During summer field campaigns, multiple measurements over a few days per year and per site are conducted with the aim to sample as much variability in stream flow as possible. At pro-glacial river sites, high diurnal discharge variability ensures usually enough points, but overcast situations (clouds) can cause low stream flow variability and can therefore can limit streamflow variability in some years.

The preparation of the FL30 measurements consists of preparing a calibration liquid to associate a dye tracer concentration with the measured signal (mV). This is achieved by preparing, first, a dilution series (see document “05 - Preparation”) of the injection color (base solution). Injected into a five-liter calibration bucket, these prepared amounts of calibration liquid then provide the basis to linearly relate the mV signal to the known dye concentration. The relationship is established in post-processing (see document “07 – Post-processing”).

### ADCP

A Teledyne Marine StreamPro ADCP of Uni Fribourg is available in Central Asia (currently at CAIAG – KG) that can be used at river sites to estimate discharge. So-obtained values have been cross-validated in 2023 with FL30 measurements and showed differences of less than 10%. For each ADCP measurement, multiple cross sections (at best under a bridge with concrete foundation, forming a square cross-section geometry), need to be recorded and evaluated in the software QRevInt (Gensis HydroTech, 2025; Mueller, 2016). The diurnal streamflow variability of larger rivers is very low in comparison to pro-glacial streams, so that multiple measurements spread over multiple years and at different seasons (summer, autumn, early winter) might be required for deriving a rating curve. Measurements at various different points in time might also show a strong deviation from previous or existing pairs of water level and discharge. This can be the result of stream bed adjustment. In such cases, a rating curve adjustment can be performed according to the outlined methodology in document “01 – Theoretical basis”.

Table 1. Specifications of the hydrological instruments

| Instrument name | Variable | Model | Manufacturer | Accuracy | Resolution |
| --- | --- | --- | --- | --- | --- |
| HOBO | Pressure [kPa, cmH2O] | U20, U20L (0-4 m) | Onset Computer Corporation, USA | 0.3 cm | 0.14 cm |
| van Essen | Pressure [kPa, cmH2O] | Diver DI801 (0-10 m) | van Essen Instruments, Netherlands | 0.5 cm | 0.3 cm |
| Fluorometer (FL30) | Dye concentration [µg l-1] | GGUN-FL30 | Albillia Sarl, Switzerland | 0.02 ppb (detection limit) | 0.1 ppb |
| ADCP | Discharge [m3 s-1] | StreamPro | Teledyne Inc., USA | ±1.0% of measurement (range: ± 5 m/s) | variable |

### Data format

For data storage and archiving, one should choose a fixed file format and structure. For the hydrological data storage, Uni Fribourg uses simple text files (CSV format) with relevant units and timezone indicated in the column names. All data are compiled into a single file per station. Timestamps are standardized to Coordinated Universal Time (UTC+0) and formatted as YYYY-MM-DDTHH:MM:SS+00:00, conform with the standard ISO8601. The file structure is provided in Table 3.

Table 3: Output data column names.

| Column | Name | Column | Name |
| --- | --- | --- | --- |
| 1 | datetime (UTC+0) | 6 | wl_corr (cmH2O) |
| 2 | pressure_water (cmH2O) | 7 | wl_final (cmH2O) |
| 3 | temperature_water (deg.C) | 8 | Q_mid (m^3 s^{-1}) |
| 4 | pressure_atmo (cmH2O) | 9 | Q_low (m^3 s^{-1}) |
| 5 | temperature_atmo (deg.C) | 10 | Q_high (m^3 s^{-1}) |

Data files before the final result will be in various different formats. Usually each company uses their own format and structure. During the processing of obtained measurements (“08 – Post-processing”), the individual file formats and structures are introduced.